THE 2026 SDR RECEIVER PROJECT

A Practical Construction Series for All Skill Levels for workshop consolidation

Aims:

- Build a complete HF SDR receiver (0–30 MHz)

- Learn breadboarding, soldering, KiCad PCB design, and enclosure building

- Explore RF basics, noise, filtering, and digital decoding

- An all ability, hands-on project aligned with amateur-radio values of construction, experimentation, and operating

PROJECT OVERVIEW

Introductory Session — 21 January 2026

Live demo, sign-up/commitment, overview of the build series.

Session A — Breadboard Build & Firmware Flashing

Wednesday 18 February 2026 — 19:00–21:00

- Assemble Tayloe detector + op-amp

- Wire Pico/Pico-2, OLED, encoder

- Flash UF2 firmware

- First HF signals received

Session B — Component Choices & Performance Testing

Wednesday 19 March 2026 — 19:00–21:00

- Compare op-amps, noise floor, Pico vs Pico-2

- HF noise behaviour

- Simple measurement exercises

Session C — KiCad PCB Design Workshop

Wednesday 16 April 2026 — 19:00–21:00

- Convert schematic to KiCad

- Place footprints, route board

- Add optional enhancements

- Export Gerbers & BOM

Session D — PCB Assembly, Enclosure & Final Testing

Wednesday 21 May 2026 — 19:00–21:00

- Soldering workshop

- Fit enclosure (3D-printed or laser-cut)

- Final alignment & listening session

- Optional: WEFAX or FT8 decoding

BOM SHEET

SEMARC SDR Receiver — Parts List

(UK Suppliers)

(Quantities are per-builder unless noted)

Core Build

| Item | Supplier | SKU / Notes |
| --- | --- | --- |
| Raspberry Pi Pico or Pico-2 | Pimoroni / The Pi Hut | Standard Pico SKU |
| MCP6022 Dual Op-Amp (DIP-8) | Farnell | 1332125 |
| 74CBTLV3253 Analog Switch | Farnell | 3869610 |
| 128×64 I2C OLED Display (SSD1306) | Amazon UK | 0.96" OLED listings |
| Rotary Encoder (EC11 type) | RS | 737-7767 |
| 3.5mm Stereo PCB Jack | RS | 476-328 |
| BNC Panel-Mount Connector | Amazon UK | Standard BNC panel-mount |
| Male Header Set for Pico | The Pi Hut | 103529 |
| Breadboard + Jumper Wires | Amazon UK | Standard kits |

## Optional Build

| Item | Supplier | SKU / Notes |
| --- | --- | --- |
| PAM8403 Audio Amplifier Module | Amazon UK | PAM8403 mini board |
| Small 4–8 Ω Speaker | Amazon UK | 3–5 W micro speaker |
| 3×AAA Battery Holder | CPC / Amazon | Standard holder |
| 3D-Printed Enclosure | Members? | STL provided in Session D |

## Passive Components (per builder)

- Resistors: 100 Ω, 1 k, 10 k, 100 k

- Capacitors: 100 nF, 10 µF electrolytic

- Inductors: as per Tayloe detector spec

- Misc: knobs, standoffs, screws, heatshrink

KICAD STARTER TEMPLATE (SESSION C)

## Schematic Blocks to Include

- Raspberry Pi Pico / Pico-2 (2×20 header)

- 74CBTLV3253 Tayloe detector

- MCP6022 op-amp (DIP-8 footprint)

- OLED (4-pin I2C header)

- Rotary encoder + push switch

- Audio output (3.5mm jack)

- Optional PAM8403 amplifier

- Power input (3×AAA or DC jack)

- Mounting holes (M3)

## PCB Layout Guidance

- 2-layer board with ground pour

- Keep RF traces short

- Place Pico centrally

- Add optional SMD pads for future upgrades

- Provide clear silkscreen labels

- Export: Gerbers, drill files, BOM, pick-and-place

Enclosure Design – Needs Work

Purpose

Practical, robust, and easy-to-assemble enclosure for the SEMARC SDR receiver PCB.

Suitable for 3D printing on common printers (PLA or PETG) and allow for optional upgrades such as a speaker and amplifier.

Design

- Protect the PCB and components

- Provide good ergonomics for tuning and viewing the display

- Allow airflow around the Pico and op-amp

- Support both headphone-only and speaker-equipped builds

- Be printable on standard 200×200 mm beds

- Require no supports where possible

- Use simple fasteners (M3 screws)

Requirements

Front Panel

- Cut-out for 128×64 OLED

- Hole for rotary encoder + knob

- Optional grille for small speaker (40–50 mm)

Rear Panel

- BNC antenna connector

- 3.5mm audio jack

- Optional power jack (if not using batteries)

Internal Layout

- PCB mounting posts (M3)

- Space for Amp module

- Space for 3×AAA battery holder or Li-ion cell (optional)

- Cable routing channels for neat wiring

Assembly

- Two-part clamshell or base + lid design

- Four M3 screws for closure

- Snap-fit options acceptable if robust

- Tolerance: 0.2–0.3 mm typical for PLA

Cosmetics Considerations

- SEMARC logo ????????   How?????

- Smooth or textured finish

- Optional colours for parts (e.g., black body, blue front panel)

- Rounded edges for comfort???

Print Recommendations

- Material: PLA for ease, PETG for durability ?????

- Layer height: 0.2 mm

- Infill: 20–30%

- Walls: 2–3 perimeters

- No supports unless absolutely necessary

- Print time: 3–6 hours depending on design

Deliverables

- STL files for:

  - Base

  - Lid

  - Front panel insert (optional)

  - Speaker grille (optional)

- Assembly diagram

- Screw list

- Printing notes

# POTENTIAL SDR PROJECT PLAN

# (2-HOUR MEETINGS)

Purpose:

A practical, inclusive project teaching construction, RF basics, PCB design, and enclosure building — naturally aligned with amateur-radio values of construction, experimentation, and operating.

Session A:

Build breadboard SDR, flash firmware, receive first signals.

Session B:

Compare op-amps, noise floor, Pico vs Pico-2, HF noise understanding.

Session C:

Convert schematic to KiCad PCB, route board, generate Gerbers.

Session D:

Assemble PCB, fit enclosure, final testing, optional WEFAX/FT8 decoding.

# COMMITTEE BRIEF

SEMARC SDR Project

Summary

In order to engage with the chosen project I suggest it becomes a four-meeting practical project to design and build a Pi-Pico-based HF SDR receiver based on the 101things idea.

As our meetings are 2 hours and we want them to be suitable for all skill levels. I think we should regard it in terms of supporting: construction, experimentation, and operating.

With careful buying (bulk?) the estimated cost per member: £15–£25 depending on options.

Would club funds allow for a small bulk-buy of components?  We should have access to soldering stations, and optional 3D-printing support.

Do we make 21 January 2026 an introductory session or start straight away?

Do we make each session sequential, or do we break them up with other meetings to give flexibility and a change especially if we have members who do not want to build this project?

I have a draft of a member’s e-mail

Subject: SEMARC Project 2026 — Build Your Own HF SDR Receiver!

Dear Members, As discussed and voted on at our last meeting we are launching a hands-on club project for 2026: designing and building a Pi-Pico-based HF SDR receiver covering 0–30 MHz.

It is low-cost, beginner-friendly, and a great way to develop practical radio skills.

Our first session will be on Wednesday 21 January 2026 at 19:30, where we’ll demo the receiver, explain the build process, and get you to “sign-up” for any potential group buy of parts.  We hope to complete the project over about four-meeting/workshops series.   This is an all skills project and is designed to consolidate and bring together some of our previous standalone workshops on design, simulation, construction and workshop practices.

If you feel the project is too difficult for you … then be assured that it will be properly supported.   If you feel the project is too simple for you then please bring your skills and expertise to help develop the skills of other members.

All welcome — bring your curiosity!